Performance Analysis of FM System under Gaussian White Noise using MATLAB

AIM :

 To simulate an FM system using MATLAB and analyze its performance under Gaussian white noise.

 To study the effect of noise on the FM signal and evaluate the output signal-to-noise ratio (SNR).

# CODE:

clc;

clear;

close all;

Am = 1;                 % Message signal amplitude

fm = 100;                % Message frequency (Hz)

fc = 1000;               % Carrier frequency (Hz)

fs = 20000;              % Sampling frequency

t = 0:1/fs:0.05;          % Time vector

kf = 200;                % Frequency sensitivity

SNR = 10;                % Signal-to-noise ratio (dB)

% Message Signal

m = Am*cos(2*pi*fm*t);

% FM Modulation

int_m = cumsum(m)/fs;

s = cos(2*pi*fc*t + 2*pi*kf*int_m);

% Add Gaussian White Noise

r = awgn(s, SNR, 'measured');

% FM Demodulation

analytic_signal = hilbert(r);

inst_phase = unwrap(angle(analytic_signal));

inst_freq = [diff(inst_phase)*fs/(2*pi), 0];

% Recover Message Signal

demod = (inst_freq - fc)/kf;

% Remove DC component

demod = demod - mean(demod);

% Calculate Signal-to-Noise Ratio

m = m - mean(m);

demod = demod - mean(demod);

error_signal = m - demod;

output_SNR = 10*log10(mean(m.^2)/mean(error_signal.^2));

% Plot Results

figure;

subplot(4,1,1);

plot(t,m);

title('Original Message Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

subplot(4,1,2);

plot(t,s);

title('FM Modulated Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

subplot(4,1,3);

plot(t,r);

title('FM Signal with Gaussian White Noise');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

subplot(4,1,4);

plot(t,demod);

title('Demodulated Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

fprintf('Input SNR = %.2f dB\n', SNR);

fprintf('Output SNR = %.2f dB\n', output_SNR);

# OUTPUT:

# RESULT :

 The FM signal was successfully generated, and Gaussian white noise was added using MATLAB.

 The noisy signal was demodulated, and its performance was analyzed by comparing the input and output SNR.