Simulation and Analysis of Uniform and NonUniform Quantization Using MATLAB

AIM :

  To simulate uniform and non-uniform quantization techniques using MATLAB.

  To calculate and compare the Signal-to-Quantization-Noise Ratio (SQNR) of both techniques.

CODE:

clc;

clear;

close all;

% Parameters

N = 1000;

t = linspace(0, 1, N);

x = sin(2*pi*5*t);

L = 8;                    % Number of quantization levels

% Uniform Quantization

xmin = -1;

xmax = 1;

delta = (xmax - xmin)/L;

x_uniform = xmin + delta/2 + ...

delta*floor((x - xmin)/delta);

x_uniform(x_uniform > xmax - delta/2) = xmax - delta/2;

x_uniform(x_uniform < xmin + delta/2) = xmin + delta/2;

% Non-Uniform Quantization (Mu-law Companding)

mu = 255;

% Compress the signal

x_compressed = sign(x).* ...

log(1 + mu*abs(x))/log(1 + mu);

% Quantize the compressed signal

delta_mu = 2/L;

xq_compressed = -1 + delta_mu/2 + ...

delta_mu*floor((x_compressed + 1)/delta_mu);

xq_compressed = min(max(xq_compressed, ...

-1 + delta_mu/2), 1 - delta_mu/2);

% Expand the quantized signal

x_nonuniform = sign(xq_compressed).* ...

((1 + mu).^abs(xq_compressed) - 1)/mu;

% Quantization Errors

error_uniform = x - x_uniform;

error_nonuniform = x - x_nonuniform;

% SQNR Calculation

SQNR_uniform = 10*log10( ...

sum(x.^2)/sum(error_uniform.^2));

SQNR_nonuniform = 10*log10( ...

sum(x.^2)/sum(error_nonuniform.^2));

% Plot Results

figure;

subplot(3,1,1);

plot(t,x,'b');

title('Original Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

subplot(3,1,2);

stairs(t,x_uniform,'r');

hold on;

plot(t,x,'b');

title('Uniform Quantization');

xlabel('Time (s)');

ylabel('Amplitude');

legend('Quantized','Original');

grid on;

subplot(3,1,3);

plot(t,x,'b');

hold on;

plot(t,x_nonuniform,'r');

title('Non-Uniform Quantization (Mu-law)');

xlabel('Time (s)');

ylabel('Amplitude');

legend('Original','Quantized');

grid on;

% Display Results

fprintf('Uniform Quantization SQNR = %.2f dB\n', ...

SQNR_uniform);

fprintf('Non-Uniform Quantization SQNR = %.2f dB\n', ...

SQNR_nonuniform);

# OUTPUT:

# RESULT :

 The FM signal was successfully generated, and Gaussian white noise was added using MATLAB.

 The noisy signal was demodulated, and its performance was analyzed by comparing the input and output SNR.